Code for the MASTER SAFETY SYSTEM
function [status_code, fuel_enable] = Global_Safety_Controller(global_risk, box_P, box_T)

%% 1. DEFINE CRITICAL SAFETY THRESHOLDS

% These values represent the physical limits for a confined hydrogen space.

P_critical = 3.5;   % Red Alert if pressure exceeds 3.5 Bar (structural risk)

P_warning  = 2.0;   % Yellow Alert if pressure exceeds 2.0 Bar

T_critical = 323.15; % Red Alert if temp exceeds 50°C (323.15 K) (fire risk)

T_warning  = 308.15; % Yellow Alert if temp exceeds 35°C (308.15 K)

H2_critical = 15;

H2_warning  = 8;

%% 2. EVALUATE EMERGENCY SHUTDOWN (RED STATE)

% Logic: If ANY sensor hits a critical limit, shut down immediately.

if (global_risk >= H2_critical) || (box_P >= P_critical) || (box_T >= T_critical)

status_code = 2; % Red Lamp: System Locked

fuel_enable = 0; % Cut off hydrogen supply

%% 3. EVALUATE WARNING STATE (YELLOW STATE)

% Logic: If we haven't hit Red, check if any sensor is in the Warning zone.

elseif (global_risk >= H2_warning) || (box_P >= P_warning) || (box_T >= T_warning)

status_code = 1; % Yellow Lamp: Maintenance Required

fuel_enable = 1; % Flow continues to allow controlled shutdown

%% 4. NORMAL OPERATION (GREEN STATE)

else

status_code = 0; % Green Lamp: System Healthy

fuel_enable = 1; % Normal Flow

end

end

CODE FOR ENERGY CONSUMPTION

function [V_voltage, I_current] = Hardware_Power_Behavior(Status_Code)

% 1. REGULATED VOLTAGE BUS

V_voltage = 5.0;

% 2. TOTAL CONTINUOUS BASELINE CURRENT (Always powered on)

% MQ-8 Heater (150mA) + DHT22 (1.5mA) + RGB LED (20mA) + Current/Volt Sensors (10.2mA)

I_continuous = 0.150 + 0.0015 + 0.020 + 0.0102;

% 3. STATE-MACHINE DRIVEN ACTUATOR CURRENT DRAW

switch Status_Code

case 0

% Nominal State: Only monitoring hardware runs

I_actuators = 0.0;

case 1

% Warning State: Relay isolates system + Cooling Fan evacuates gas

% Relay Coil (70mA) + 5V DC Fan (200mA)

I_actuators = 0.070 + 0.200;

case 2

% Critical State: Relay and Fan stay active + Emergency Buzzer sounds

% Relay Coil (70mA) + 5V DC Fan (200mA) + Buzzer (30mA)

I_actuators = 0.070 + 0.200 + 0.030;

otherwise

% Safety Fallback: Assume worst-case load if status code is undefined

I_actuators = 0.070 + 0.200 + 0.030;

end

% 4. COLLECTIVE POWER BUS CURRENT DRAW (Amperes)

I_current = I_continuous + I_actuators;

end

Code for STORAGE Or TANK

function [P_tank, T_tank, m_dot_out] = tank_logic(q_demand)

% Parameters

persistent m_current T_current

% INITIALIZATION

if isempty(m_current)

% considerable pressure (e.g., ~350 bar):

% P = (m*R*T)/V -> m = (P*V)/(R*T)

% (350e5 * 0.1) / (4124 * 293.15) approx 29 kg

m_current = 29; % Initial mass in kg for high pressure

T_current = 293.15; % Initial Temp in Kelvin (20°C)

end

% TANK CONSTANTS

V = 0.5;    % Tank Volume (m3)

R = 4124;   % Gas constant for H2 (J/kg.K)

dt = 0.01;  % Simulation step size

% MASS BALANCE (REFILL REMOVED)

% The tank only depletes based on the demand (q_demand)

m_dot_net = - q_demand;

m_current = m_current + (m_dot_net * dt);

% PRESSURE DYNAMICS (Ideal Gas Law)

% Result will be in Pascals (Pa)

P_tank = (m_current * R * T_current) / V;

% OUTPUTS

T_tank = T_current;

m_dot_out = q_demand;

end